Hardware Used

The current prototype is built using the following hardware components:

| Component | Model | Purpose |
| --- | --- | --- |
| Microcontroller | ESP32-S3 N16R8 | Main controller responsible for audio recording, storage, WiFi, BLE, and cloud communication |
| Microphones | 2 × INMP441 Digital MEMS Microphones | Capture high-quality digital audio (supports stereo or mono recording) |
| Storage | FAT32 MicroSD Card | Stores recorded WAV audio files and pending upload queue |
| Status Indicator | WS2812B RGB LED | Displays recording, upload, WiFi, battery, and error status |
| Start Button | Tactile Push Button | Starts audio recording |
| Stop Button | Tactile Push Button | Stops recording and initiates file saving/upload |
| Battery | 3.7V Li-ion Battery | Portable power source |
| Battery Monitoring | ADC Voltage Divider | Measures battery voltage and remaining charge |
| USB Interface | USB-C (USB-CDC) | Device programming, debugging, and serial configuration |

Current Firmware Functionality

The current firmware (v1.7.4) supports the following features:

Audio Recording

- Record audio using dual INMP441 digital microphones.

- Save recordings in 16-bit PCM WAV format.

- Support configurable sample rates (default: 16 kHz).

- Support stereo, left-only, or right-only recording modes.

Local Storage

- Save recordings to a microSD card.

- Generate timestamp-based filenames.

- Verify file integrity after recording.

- Maintain a queue of recordings waiting for upload.

AWS Cloud Upload

- Automatically upload recordings to Amazon S3 using secure HTTPS.

- Obtain presigned upload URLs before uploading.

- Retry failed uploads automatically.

- Delete successfully uploaded files from the SD card.

Bluetooth Low Energy (BLE)

- Control the recorder using a mobile application.

- Supported BLE commands:

  - Start recording

  - Stop recording

  - Get device status

  - Configure WiFi

  - Clear WiFi credentials

- Send live device status notifications, including recording state, battery level, WiFi status, firmware version, and pending uploads.

WiFi Management

- Connect to WiFi in Station mode.

- Store up to three WiFi networks.

- Automatically reconnect to known networks.

- Synchronize device time using NTP.

Battery Monitoring

- Continuously monitor battery voltage.

- Calculate battery percentage.

- Display low-battery warnings using the RGB LED.

LED Status Indication

The RGB LED provides visual feedback for device status:

- Red: Recording

- Red Blinking: Saving recording

- Green Blinking: Upload in progress

- Green: Upload successful

- Blue: WiFi connected

- Blue Blinking: Connecting to WiFi

- Amber: Low battery or system error

- White: Idle state

Serial Console

The firmware provides a USB serial interface for:

- Starting and stopping recordings

- Configuring WiFi

- Viewing device status

- Changing audio parameters

- Running SD card diagnostics

- Saving or restoring device settings

Persistent Configuration

The following settings are stored in ESP32 Non-Volatile Storage (NVS):

- Sample rate

- Audio format

- Audio channel mode

- Upload enable/disable setting

- Recording counter

- Saved WiFi credentials

Current System Workflow

- User starts recording using the button, BLE, or serial command.

- Audio is captured from the dual MEMS microphones.

- Audio is converted into 16-bit PCM WAV format.

- The recording is saved to the microSD card.

- If WiFi is available, the firmware requests a presigned URL and uploads the recording to Amazon S3.

- If the upload fails, the recording is added to a pending queue and retried automatically once connectivity is restored.