MinuteX Device Pairing & Cloud Architecture (MVP)

1. Device Identity

Every device is provisioned during manufacturing with:

- Device ID (Public & Permanent)

- Device API Key (Secret & Permanent)

- QR Code (contains Device ID or Device ID + Serial Number)

These never change during the lifetime of the device.

2. Data Ownership

The User owns all recordings and AI data, not the device.

Store:

S3

recordings/{user_id}/{recording_id}.wav

DynamoDB

Recording

├── recording_id

├── user_id

├── device_id

├── transcript

├── summary

└── metadata

Every recording stores the originating Device ID for auditing.

3. Device Model

- One user can own multiple devices.

- One device can be paired with only one user at a time.

- Multi-user devices are not supported in the MVP.

4. Pairing Flow

Step 1

User taps

Add Device

Step 2

User scans the QR Code
(or manually enters the Device ID / Serial Number).

Step 3

App requests pairing.

Backend generates a

6-digit pairing code

(valid for 5 minutes).

Step 4

User enters the pairing code on the recorder.

Step 5

Device sends

- Device ID

- Device API Key

- Pairing Code

Backend verifies:

- Device exists

- API Key matches

- Pairing code is valid

Step 6

Backend links

Device

↓

User

Device becomes

PAIRED

5. Recording Upload Flow

Whenever Wi-Fi is available:

Device uploads

- Device ID

- Device API Key

- Recording ID

- Audio File

Backend

Device ID

↓

Find paired user

↓

Store in S3

↓

Trigger AI Processing

↓

Save metadata

The app fetches recordings by User ID.

6. Offline Recording

Phone is not required.

Flow

Record Offline

↓

Save Locally

↓

Wi-Fi Available

↓

Auto Upload

↓

Processing

↓

Visible in App

7. Bluetooth & Connectivity

Bluetooth is only for communication.

It is NOT the pairing mechanism.

If Bluetooth disconnects because

- Phone moved away

- App closed

- Bluetooth turned off

- Phone battery died

Nothing happens.

Device remains paired.

Recording continues.

Uploads continue once Wi-Fi becomes available.

No re-pairing required.

8. Device Dashboard (BLE Connected)

When BLE is connected, the app should show live device information.

Example:

MinuteX Recorder

Status: Connected

Battery: 82%

Wi-Fi: Connected

Storage:

6.2 GB / 16 GB

Pending Recordings:

8

Pending Upload Size:

246 MB

Upload Progress:

Meeting_004.wav (45%)

Currently Recording:

Yes

Firmware:

v1.0.3

Last Sync:

Just now

9. Recording Queue

BLE can expose

- Number of pending recordings

- Pending upload size

- Upload progress

- Currently uploading file

- Current recording status

This gives users confidence that recordings are syncing correctly.

10. Unpair

Unpair only removes the account association.

Keeps

- Device ID

- Device API Key

- Wi-Fi credentials

- Device settings

- Firmware

Removes

- Paired User

Device cannot upload after unpairing.

Cloud recordings remain safe.

11. Better Unpair UX

Before removing the device, the app checks over BLE:

Pending recordings:

8

Pending upload size:

246 MB

Then displays

This device has 8 recordings waiting to be uploaded (246 MB). What would you like to do?

Options

- Upload & Remove

- Remove Now

- Cancel

Upload & Remove

Upload Remaining Files

↓

Verify Upload Success

↓

Backend Unpairs Device

Remove Now

Immediate Unpair

↓

Pending recordings stay on device

↓

Cannot upload anymore

This avoids accidental data loss while giving the user full control.

12. Factory Reset

Factory Reset returns the recorder to an out-of-box state.

Deletes

- Wi-Fi credentials

- Pairing information

- Cached tokens

- User settings

- Local recordings (optional based on product decision)

Keeps

- Device ID

- Device API Key

- Firmware

- QR Code

- Serial Number

Cloud recordings remain in the user's account.

13. Re-Pair

After

- Unpair

- Factory Reset

Pairing is exactly the same.

Scan QR

↓

Receive Pairing Code

↓

Enter Code

↓

Authenticate

↓

Device Paired

No new Device ID is created.

14. QR Code Damage

If the QR code becomes unreadable, the app should also support:

- Manual Device ID entry

- Manual Serial Number entry

The device should have both printed on its body.

Example:

Device ID:

DEV-ABC123456

Serial No:

SN00012345

15. Device States

Manufactured

│

▼

UNPAIRED

│

▼

Pair Request

│

▼

Pairing Code Verified

│

▼

PAIRED

│

├── Record

├── Upload

├── AI Processing

├── BLE Sync

└── OTA Updates

│

├─────────────┐

│             │

▼             ▼

UNPAIR      FACTORY RESET

│             │

▼             ▼

UNPAIRED     UNPAIRED

16. Core Principles

- User owns all recordings and AI data.

- Device is authenticated using Device ID + Device API Key.

- Device ID is permanent.

- Device API Key is permanent.

- Bluetooth is only for communication and setup—not ownership.

- Recording works completely offline.

- Wi-Fi is only required for uploads.

- Cloud is the source of truth.

- BLE provides live device status (battery, storage, pending recordings, upload progress, firmware, etc.).

- Unpair removes only the account association.

- Factory Reset erases local device data but preserves the device's permanent identity.

- Users should be warned about pending recordings before unpairing and given the choice to upload them first or remove the device immediately.

- QR code damage should not prevent pairing because the Device ID and Serial Number can be entered manually.